| Exp No: 10 | Internetworking with routers in CISCO PACKET TRACER simulator |
| --- | --- |

# AIM:

To design and configure internetworks in Cisco Packet Tracer using (a) a router with static IP addresses, (b) a wireless router with DHCP server and an internet cloud, and (c) a switch, a router and Ethernet cables in the lab, and to trace how the packets are routed.

# ALGORITHM / PROCEDURE:

## Simple internetwork using a router

  - Place a Router (2911), two Switches and six PCs (3 in each network) on the workspace.

  - Connect each PC to its switch and each switch to the router with copper straight-through cables. Router interface Gig0/0 goes to Switch0 and Gig0/1 goes to Switch1.

  - Assign the static IP addresses to the PCs (network 1: 192.168.1.0/24, network 2: 192.168.2.0/24) and set the default gateway of each PC to the router interface address (192.168.1.1 / 192.168.2.1).

  - Open the router CLI and configure both interfaces with ip address and no shutdown (commands given below).

  - Ping from PC0 to PC3 and check the reply. Use tracert to see the path.

  - Go to Simulation mode, add a simple PDU from PC0 to PC3 and trace the packet through Switch0, Router0 and Switch1.

## Wireless router, DHCP server and internet cloud

  - Place a Wireless Router, a Cable Modem, a Cloud, a Server, one PC (wired), a Laptop and a Smartphone.

  - Connect PC0 to a LAN port of the wireless router, the Internet port of the router to the Cable Modem, the modem to the Cloud and the Cloud to the Server (Coaxial / Ethernet).

  - Open the wireless router GUI: set SSID = LabNet, WPA2-PSK password and Internet setup = DHCP. Set the DHCP server = Enabled (start IP 192.168.0.100, 50 users).

  - Give the server a static IP 203.0.113.10. On the Laptop and the Smartphone configure the Wireless0 as SSID LabNet with the same password.

  - In IP Configuration select DHCP on the PC, Laptop and Smartphone and check the address received.

  - Ping the server and also open its web page from the browser of the Laptop.

## Inter-network in the lab using switch, router and Ethernet cables

  - Take one router, two switches, six PCs and eight straight-through UTP cables (made in Experiment 2).

  - Connect PCs to the switches, and the switches to the two Gigabit ports of the router. Check that all the link LEDs glow.

  - Assign the addresses 172.16.1.0/24 and 172.16.2.0/24 with the router as the default gateway and configure the router interfaces.

  - Ping within the same network and between the two networks. Note down the results.

| Connection | Cable used |
| --- | --- |
| PC to Switch | Straight-through |
| Switch to Router | Straight-through |
| PC to Router (direct) | Crossover |

| Connection | Cable used |
| --- | --- |
| Router to Router (LAN side) | Crossover |

## Router configuration (Part a)

Router CLI

Router>enable Router#configure terminal Router(config)#hostname R1 R1(config)#interface gigabitEthernet 0/0 R1(config-if)#ip address 192.168.1.1 255.255.255.0 R1(config-if)#no shutdown R1(config-if)#exit R1(config)#interface gigabitEthernet 0/1 R1(config-if)#ip address 192.168.2.1 255.255.255.0 R1(config-if)#no shutdown R1(config-if)#end R1#copy running-config startup-config

# DIAGRAM:

R

Router0

Gig0/0: 192.168.1.1

Gig0/1: 192.168.2.1

Switch0

Switch1

PC

PC

PC

PC

PC

PC

SW

SW

PC0

192.168.1.2

PC1

192.168.1.3

PC2

192.168.1.4

PC3

192.168.2.2

PC4

192.168.2.3

PC5

192.168.2.4

Fig 1: Internetwork of two LANs connected by a router (Part a)

Internet

Wireless Router

192.168.0.1

Cable Modem

Cloud0

Server0

203.0.113.10

PC

PC

PC

HUB

WR

PC0

Laptop0

Smartphone0

Fig 2: Wireless router with DHCP and internet cloud (Part b) - dotted lines are wireless links

R

Router

Gig0/0: 172.16.1.1

Gig0/1: 172.16.2.1

Switch-A

Switch-B

PC

PC

PC

PC

PC

PC

SW

SW

PC1

172.16.1.2

PC2

172.16.1.3

PC3

172.16.1.4

PC4

172.16.2.2

PC5

172.16.2.3

PC6

172.16.2.4

Fig 3: Lab inter-network using switches, router and Ethernet cables (Part c)

# OUTPUT:

## Part (a) - Router with two networks

| Cisco Packet Tracer - internetwork_router.pkt |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Router0 |  |  |  |
|  |  |  | Gig0/0: 192.168.1.1 |  |  |  |
|  |  |  | Gig0/1: 192.168.2.1 |  |  |  |
|  | SW |  |  |  | SW |  |
|  | Switch0 |  |  |  | Switch1 |  |
| PC0 | PC1 | PC2 |  | PC3 | PC4 | PC5 |
| 192.168.1.2 | 192.168.1.3 | 192.168.1.4 |  | 192.168.2.2 | 192.168.2.3 | 192.168.2.4 |

Packet Tracer view of Part (a)

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP Gateway of last resort is not set

192.168.1.0/24 is variably subnetted, 2 subnets, 2 masks

C	192.168.1.0/24 is directly connected, GigabitEthernet0/0 L	192.168.1.1/32 is directly connected, GigabitEthernet0/0

192.168.2.0/24 is variably subnetted, 2 subnets, 2 masks

YES unset administratively down down

Protocol up

up

OK? Method Status YES manual up YES manual up

IP-Address 192.168.1.1

192.168.2.1

unassigned

Interface GigabitEthernet0/0 GigabitEthernet0/1 Vlan1

R1#show ip route

R1(config-if)#no shutdown

%LINK-5-CHANGED: Interface GigabitEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0, changed state to up R1(config-if)#end

R1#show ip interface brief

R1 - CLI

Trace complete.

C:\>ping 192.168.2.3

Pinging 192.168.2.3 with 32 bytes of data: Request timed out.

Reply from 192.168.2.3: bytes=32 time=2ms TTL=127 Reply from 192.168.2.3: bytes=32 time=1ms TTL=127 Reply from 192.168.2.3: bytes=32 time=1ms TTL=127

Ping statistics for 192.168.2.3:

Packets: Sent = 4, Received = 3, Lost = 1 (25% loss), Approximate round trip times in milli-seconds:

Minimum = 0ms, Maximum = 2ms, Average = 1ms C:\>tracert 192.168.2.3

Tracing route to 192.168.2.3 over a maximum of 30 hops:

PC0 - Command Prompt

192.168.2.0/24 is directly connected, GigabitEthernet0/1 192.168.2.1/32 is directly connected, GigabitEthernet0/1

C L

R1 - CLI (contd.)

| 1 | 0 ms | 0 ms | 0 ms | 192.168.1.1 |
| --- | --- | --- | --- | --- |
| 2 | 0 ms | 0 ms | 1 ms | 192.168.2.3 |

| Fire | Last Status | Source | Destination | Type | Color | Time(sec) | Periodic | Num |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Successful | PC0 | Switch0 | ICMP |  | 0.000 | N | 0 |
|  | Successful | Switch0 | Router0 | ICMP |  | 0.001 | N | 1 |
|  | Successful | Router0 | Switch1 | ICMP |  | 0.002 | N | 2 |
|  | Successful | Switch1 | PC3 | ICMP |  | 0.003 | N | 3 |
|  | Successful | PC3 | Switch1 | ICMP |  | 0.004 | N | 4 |
|  | Successful | Switch1 | Router0 | ICMP |  | 0.005 | N | 5 |
|  | Successful | Router0 | Switch0 | ICMP |  | 0.006 | N | 6 |
|  | Successful | Switch0 | PC0 | ICMP |  | 0.007 | N | 7 |

The router receives the frame on Gig0/0, removes the Ethernet header, looks at the destination IP in the routing table, and sends a new frame out of Gig0/1 to PC3.

## Part (b) - Wireless router, DHCP and internet cloud

Internet

Wireless Router0

192.168.0.1

Cable Modem

Cloud0

Server0

203.0.113.10

PC0

192.168.0.100

Laptop0

192.168.0.101

Smartphone0

192.168.0.102

Cisco Packet Tracer - wireless_dhcp.pkt

HUB

WR

Packet Tracer view of Part (b)

C:\>ipconfig /all

Wireless0 Connection:(default port) Connection-specific DNS Suffix..:

Physical Address................: 0001.C7AB.4D12

Link-local IPv6 Address.........: FE80::201:C7FF:FEAB:4D12 IPv6 Address....................: ::

IP Address......................: 192.168.0.101

Subnet Mask.....................: 255.255.255.0

Default Gateway.................: 192.168.0.1

DHCP Servers....................: 192.168.0.1

DNS Servers.....................: 0.0.0.0

C:\>ping 203.0.113.10

Pinging 203.0.113.10 with 32 bytes of data:

Reply from 203.0.113.10: bytes=32 time=21ms TTL=126 Reply from 203.0.113.10: bytes=32 time=12ms TTL=126 Reply from 203.0.113.10: bytes=32 time=14ms TTL=126 Reply from 203.0.113.10: bytes=32 time=11ms TTL=126

Ping statistics for 203.0.113.10:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds:

Minimum = 11ms, Maximum = 21ms, Average = 14ms

Laptop0 - Command Prompt

| Device | Connection | Method | IP address received | Ping to server |
| --- | --- | --- | --- | --- |
| PC0 | Wired (LAN port) | DHCP | 192.168.0.100 | Successful |
| Laptop0 | Wireless (LabNet) | DHCP | 192.168.0.101 | Successful |
| Smartphone0 | Wireless (LabNet) | DHCP | 192.168.0.102 | Successful |

## Part (c) - Lab network with switches, router and Ethernet cables

PC1 - Command Prompt

C:\>ipconfig

FastEthernet0 Connection:(default port)

IP Address......................: 172.16.1.2

Subnet Mask.....................: 255.255.255.0

Default Gateway.................: 172.16.1.1

C:\>ping 172.16.1.3

Pinging 172.16.1.3 with 32 bytes of data:

Reply from 172.16.1.3: bytes=32 time=1ms TTL=127 Reply from 172.16.1.3: bytes=32 time=<1ms TTL=127 Reply from 172.16.1.3: bytes=32 time=<1ms TTL=127 Reply from 172.16.1.3: bytes=32 time=<1ms TTL=127

Ping statistics for 172.16.1.3:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds:

Minimum = 0ms, Maximum = 2ms, Average = 1ms C:\>ping 172.16.2.4

Pinging 172.16.2.4 with 32 bytes of data:

Request timed out.

Reply from 172.16.2.4: bytes=32 time=2ms TTL=127 Reply from 172.16.2.4: bytes=32 time=1ms TTL=127 Reply from 172.16.2.4: bytes=32 time=1ms TTL=127

Ping statistics for 172.16.2.4:

Packets: Sent = 4, Received = 3, Lost = 1 (25% loss), Approximate round trip times in milli-seconds:

Minimum = 0ms, Maximum = 2ms, Average = 1ms

| Source | Destination | Same network? | Through router? | Result |
| --- | --- | --- | --- | --- |
| PC1 (172.16.1.2) | PC3 (172.16.1.4) | Yes | No | Successful |
| PC1 (172.16.1.2) | PC4 (172.16.2.2) | No | Yes | Successful |
| PC6 (172.16.2.4) | PC2 (172.16.1.3) | No | Yes | Successful |

# RESULT:

Thus the internetworks were designed and configured in Cisco Packet Tracer using a router with static IP addresses, a wireless router with DHCP server and internet cloud, and a lab network using switches, a router and Ethernet cables. The packet transmission across the networks was traced in simulation mode.