| Exp No: 14 | Packet sniffing using RAW sockets |
| --- | --- |

# AIM:

To write a program using raw sockets to capture (sniff) the packets passing through a network interface and to display the Ethernet, IP and TCP / UDP / ICMP header details.

# ALGORITHM / PROCEDURE:

- Create a raw socket with socket(AF_PACKET, SOCK_RAW, ntohs(3)). This socket receives every frame (ETH_P_ALL = 3) that reaches the network card, with the Ethernet header. Root permission is required.

- Start an infinite loop and receive a frame with recvfrom() (maximum 65535 bytes).

- Unpack the first 14 bytes as the Ethernet header: destination MAC (6 bytes), source MAC (6 bytes) and the type field (2 bytes).

- If the type is 0x0800 (IPv4), unpack the next 20 bytes as the IP header and get the TTL, the protocol number, the source IP and the destination IP. The header length is found from the IHL field.

- Using the protocol number, unpack the transport header: 1 = ICMP (type, code), 6 = TCP (ports, sequence, acknowledgement), 17 = UDP (ports, length).

- Display the details of the packet. Repeat until the required number of packets (8) is captured.

- Generate some traffic in another terminal (ping, UDP echo, browser) and watch the packets being displayed. Close the socket.

| Protocol number | Protocol | Header read by the program |
| --- | --- | --- |
| 1 | ICMP | Type, Code |
| 6 | TCP | Source port, destination port, sequence no., acknowledgement no. |
| 17 | UDP | Source port, destination port, length |

# DIAGRAM:

| Ethernet header 14 bytes | IP header 20 bytes | TCP / UDP / ICMP header 20 / 8 / 8 bytes | Data (payload) remaining bytes |
| --- | --- | --- | --- |

struct "!6s6sH"	struct "!BBHHHBBH4s4s"	TCP "!HHLLB" / UDP "!HHH"

Packet received by the raw socket (AF_PACKET)

Fig 1: Structure of a captured frame and the struct format used for each header

| Network card (NIC) |  | Kernel driver |  | Raw socket (AF_PACKET) |  | Python program |  | Header details on screen |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Fig 2: Flow of packet sniffing using a raw socket

# PROGRAM:

sniffer.py

# Packet sniffer using RAW sockets (Linux) - run as root import socket, struct def mac(b): return ':'.join('%02x' % x for x in b) s = socket.socket(socket.AF_PACKET, socket.SOCK_RAW, socket.ntohs(3)) print("Sniffing started... (capturing 8 packets)\n") count = 0 while count < 8: raw, info = s.recvfrom(65535) if info[2] == socket.PACKET_OUTGOING: # skip our own outgoing copy continue dst, src, proto = struct.unpack("!6s6sH", raw[:14]) if proto != 0x0800: # only IPv4 continue count += 1 ver_ihl, tos, tlen, ident, frag, ttl, p, chk, sip, dip = struct.unpack("!BBHHHBBH4s4s", raw[14:3 4]) ihl = (ver_ihl & 15) * 4 print(f"Packet {count} [{info[0]}]") print(f" Ethernet : {mac(src)} -> {mac(dst)}") print(f" IPv4 : {socket.inet_ntoa(sip)} -> {socket.inet_ntoa(dip)} TTL={ttl} Length={tlen }") off = 14 + ihl if p == 1: t, c = struct.unpack("!BB", raw[off:off+2]) print(f" ICMP : type={t} code={c}") elif p == 6: sp, dp, seq, ack, offs = struct.unpack("!HHLLB", raw[off:off+13]) print(f" TCP : {sp} -> {dp} seq={seq} ack={ack}") elif p == 17: sp, dp, ln = struct.unpack("!HHH", raw[off:off+6]) print(f" UDP : {sp} -> {dp} length={ln}") else: print(f" Protocol : {p}") print() s.close()

# OUTPUT:

Sniffing started... (capturing 8 packets)

Packet 1 [lo]

Ethernet : 00:00:00:00:00:00 -> 00:00:00:00:00:00 IPv4	: 127.0.0.1 -> 127.0.0.1 TTL=64 Length=37 UDP	: 43740 -> 6000 length=17

Packet 2 [lo]

Ethernet : 00:00:00:00:00:00 -> 00:00:00:00:00:00 IPv4	: 127.0.0.1 -> 127.0.0.1 TTL=64 Length=37 UDP	: 6000 -> 43740 length=17

Packet 3 [lo]

Ethernet : 00:00:00:00:00:00 -> 00:00:00:00:00:00 IPv4	: 127.0.0.1 -> 127.0.0.1 TTL=64 Length=41 UDP	: 43740 -> 6000 length=21

Packet 4 [lo]

Ethernet : 00:00:00:00:00:00 -> 00:00:00:00:00:00 IPv4	: 127.0.0.1 -> 127.0.0.1 TTL=64 Length=41 UDP	: 6000 -> 43740 length=21

Packet 5 [lo]

Ethernet : 00:00:00:00:00:00 -> 00:00:00:00:00:00 IPv4	: 127.0.0.1 -> 127.0.0.1 TTL=64 Length=52

ICMP	: type=8 code=0

Packet 6 [lo]

Ethernet : 00:00:00:00:00:00 -> 00:00:00:00:00:00 IPv4	: 127.0.0.1 -> 127.0.0.1 TTL=64 Length=52

ICMP	: type=0 code=0

Packet 7 [lo]

Ethernet : 00:00:00:00:00:00 -> 00:00:00:00:00:00 IPv4	: 127.0.0.1 -> 127.0.0.1 TTL=64 Length=52

ICMP	: type=8 code=0

Packet 8 [lo]

Ethernet : 00:00:00:00:00:00 -> 00:00:00:00:00:00 IPv4	: 127.0.0.1 -> 127.0.0.1 TTL=64 Length=52

ICMP	: type=0 code=0

-VirtualBox:~$ sudo python3 sniffer.py

UuUSsEeRr	@UuUSsEeRr

UuUsSeErR @UuUSsEeRr -VirtualBox: ~ (Terminal 1)

PING 127.0.0.1 (127.0.0.1) 32 bytes of data.

32 bytes from 127.0.0.1: icmp_seq=1 ttl=64 time=0.015 ms

-VirtualBox:~$ sudo python3 myping.py 127.0.0.1

UusSseEerRr	@UuUSsEeRr

: Hello UDP

: Hello UDP

: Datagram test

: Datagram test

Echoed Sent Echoed

-VirtualBox:~$ python3 echo_udp_server.py &

-VirtualBox:~$ python3 echo_udp_client.py

@UusSseEerRr @UuUsSeErR

UuUSsEeRr uUsSseEerRr

Sent

uUsSeErR @uUsSeErR -VirtualBox: ~ (Terminal 2 - traffic generated)

| Packet | Protocol | Source | Destination | Details |
| --- | --- | --- | --- | --- |
| 1, 3 | UDP | 127.0.0.1 : 43740 | 127.0.0.1 : 6000 | Datagram from the client (17 and 21 byte payload) |
| 2, 4 | UDP | 127.0.0.1 : 6000 | 127.0.0.1 : 43740 | Echo from the server |
| 5, 7 | ICMP | 127.0.0.1 | 127.0.0.1 | Type 8 - Echo Request |
| 6, 8 | ICMP | 127.0.0.1 | 127.0.0.1 | Type 0 - Echo Reply |

The capture was taken on the loopback interface lo, so the MAC addresses are all zeros. On a real interface (enp0s3) the actual MAC addresses are shown.

# RESULT:

Thus a packet sniffer was implemented using raw sockets. The Ethernet, IP and UDP / ICMP header details of the packets passing through the interface were captured and displayed successfully.