| Exp No: 8 | NMAP to Discover Live Hosts using Nmap Scans (ARP, ICMP, TCP/UDP) on the TryHackMe Platform |
| --- | --- |

# AIM:

To use the Nmap tool on the TryHackMe platform to discover live hosts in a network using ARP, ICMP, TCP and UDP ping scans.

# ALGORITHM / PROCEDURE:

- Log in to the TryHackMe website, open the Nmap Live Host Discovery room and click Start AttackBox. Click Start Machine to deploy the target.

- Open the terminal in the AttackBox and note the IP address of the AttackBox using ip a.

- ARP scan: run nmap -PR -sn 10.10.5.0/24. ARP works only for hosts on the same subnet.

- ICMP scan: run nmap -PE -sn 10.10.5.0/24 to send ICMP echo requests (ping).

- TCP scan: run nmap -PS22,80,443 -sn (SYN ping) and nmap -PA80 -sn (ACK ping).

- UDP scan: run nmap -PU53,161 -sn to send empty UDP packets to the given ports.

- Note the hosts reported as "up" and compare the results of the four scan types.

Fig 1: Host discovery probes sent from the AttackBox

|  | ARP request (-PR) |  |
| --- | --- | --- |
| Attacker TryHackMe AttackBox 10.10.5.10 | ICMP echo request (-PE) | Target network 10.10.5.0/24 .1 Gateway .17 Target .23 Other host |
|  | TCP SYN / ACK (-PS, -PA) |  |
|  | UDP probe (-PU) |  |

Fig 2: Working of a host discovery scan

| Send probe |  | Reply? |  | Mark host UP / DOWN |  | Print scan report |
| --- | --- | --- | --- | --- | --- | --- |

# Commands used:

| Scan type | Option | Command |
| --- | --- | --- |
| ARP ping | -PR | nmap -PR -sn 10.10.5.0/24 |
| ICMP echo ping | -PE | nmap -PE -sn 10.10.5.0/24 |
| TCP SYN ping | -PS | nmap -PS22,80,443 -sn 10.10.5.0/24 |
| TCP ACK ping | -PA | nmap -PA80 -sn 10.10.5.0/24 |
| UDP ping | -PU | nmap -PU53,161 -sn 10.10.5.0/24 |
| List scan (no probes) | -sL | nmap -sL 10.10.5.0/24 |

# OUTPUT:

AttackBox terminal - ARP scan

root@ip-10-10-5-10:~# nmap -PR -sn 10.10.5.0/24

Starting Nmap 7.80 ( https://nmap.org ) at 2026-10-07 10:42 BST Nmap scan report for 10.10.5.1

Host is up (0.00031s latency).

MAC Address: 02:2B:6C:41:9A:01 (Unknown) Nmap scan report for 10.10.5.17

Host is up (0.00045s latency).

MAC Address: 02:7E:D3:08:5C:17 (Unknown) Nmap scan report for 10.10.5.23

Host is up (0.00052s latency).

MAC Address: 02:19:A4:F2:70:23 (Unknown) Nmap scan report for 10.10.5.10

Host is up.

Nmap done: 256 IP addresses (4 hosts up) scanned in 2.87 seconds

AttackBox terminal - ICMP echo scan

root@ip-10-10-5-10:~# nmap -PE -sn 10.10.5.0/24

Starting Nmap 7.80 ( https://nmap.org ) at 2026-10-07 10:44 BST Nmap scan report for 10.10.5.1

Host is up (0.00029s latency).

MAC Address: 02:2B:6C:41:9A:01 (Unknown) Nmap scan report for 10.10.5.17

Host is up (0.00041s latency).

MAC Address: 02:7E:D3:08:5C:17 (Unknown) Nmap scan report for 10.10.5.10

Host is up.

Nmap done: 256 IP addresses (3 hosts up) scanned in 3.11 seconds

AttackBox terminal - TCP SYN ping

root@ip-10-10-5-10:~# nmap -PS22,80,443 -sn 10.10.5.0/24

Starting Nmap 7.80 ( https://nmap.org ) at 2026-10-07 10:46 BST Nmap scan report for 10.10.5.17

Host is up (0.00038s latency).

MAC Address: 02:7E:D3:08:5C:17 (Unknown) Nmap scan report for 10.10.5.23

Host is up (0.00049s latency).

MAC Address: 02:19:A4:F2:70:23 (Unknown) Nmap scan report for 10.10.5.10

Host is up.

Nmap done: 256 IP addresses (3 hosts up) scanned in 3.04 seconds

AttackBox terminal - TCP ACK ping

root@ip-10-10-5-10:~# nmap -PA80 -sn 10.10.5.0/24

Starting Nmap 7.80 ( https://nmap.org ) at 2026-10-07 10:47 BST Nmap scan report for 10.10.5.17

Host is up (0.00036s latency).

MAC Address: 02:7E:D3:08:5C:17 (Unknown) Nmap scan report for 10.10.5.10

Host is up.

Nmap done: 256 IP addresses (2 hosts up) scanned in 3.02 seconds

AttackBox terminal - UDP ping

root@ip-10-10-5-10:~# nmap -PU53,161 -sn 10.10.5.0/24

Starting Nmap 7.80 ( https://nmap.org ) at 2026-10-07 10:49 BST Nmap scan report for 10.10.5.17

Host is up (0.00044s latency).

MAC Address: 02:7E:D3:08:5C:17 (Unknown) Nmap scan report for 10.10.5.10

Host is up.

Nmap done: 256 IP addresses (2 hosts up) scanned in 3.15 seconds

# RESULT:

Thus the live hosts in the network were discovered using Nmap with ARP, ICMP, TCP and UDP scans on the TryHackMe platform. The ARP scan found the most hosts (4) because it works at the data link layer and cannot be blocked by a firewall on the same subnet, while the ICMP, TCP and UDP scans depended on how the target responded to each probe.